ScAnVP (Scan Analysis and Visualization Processor) Ver. 7.0w - (C) 2016

Center for Neurosciences, Feinstein Institute for Medical Research,

Northwell Health (formerly, North Shore-Long Island Jewish Health System)

350 Community Drive, Manhasset, NY 11030.

GENERAL DESCRIPTION OF SOFTWARE

INTRODUCTION

ScAnVP is a set of MATLAB routines intended primarily for the image processing, visualization, region of interest statistics and analysis of PET functional brain images or other 	single volume images such as perfusion ASL fMRI or VBM MRI. ScAnVP was developed as an in-house package at the North Shore-LIJ Feinstein Institute (now Northwell Health) in collaboration with members of the Functional Brain imaging laboratory headed by Dr. David Eidelberg. The public release of ScAnVP software is distributed as copyright software that is free under the terms of the GNU General Public 	License version 2 as published by the Free Software Foundation (or any later version at your option). License terms can be read by invoking 'show license' on the ScAnVP menu or by visiting http://www.gnu.org/copyleft/. ScAnVP comes with ABSOLUTELY NO WARRANTY and no implied support and is not intended for commercial use.

ScAnVP currently runs primarily as a set of MATLAB scripts in a Windows PC platform and utilize a highly interactive, user-friendly GUI interface. This version of ScAnVP was tested on Windows 7 PC workstations with 4GB RAM utilizing MATLAB version 7.3 (R2006b) at 2.3GHz or greater but is not limited to these configurations. For MATLAB routines, ScAnVP assumes the ANALYZE volume file format for image input and output but nifty images in (.img, .hdr) format can also be input in most routines to provide compatibility with other standard neuroimaging software. A separate routine is provided to convert .nii type files to ANALYZE format.

The ScAnVP distribution includes in-house software, documentation, masks, color maps and executable programs. It may also include publicly distributed utilities or images such as SPM\sample mri or pet images and a modified brain atlas based on the aal_MNI_V4 image accessed by the software.

A primary component of ScAnVP implements the SSM-PCA model for regional and voxel based spatial covariance pattern derivation of 	characteristic imaging biomarkers of neurofunctional disorders such as Parkinson's disease and prospective single subject evaluation for 	differential diagnosis, treatment monitoring, and the assessment of drug efficacy.

Major features of ScAnVP are listed below. Modifications and additions in sequential versions are listed in the file 'vp_revisions'.

Send comments to: pspetsie@northwell.edu

INSTALLATION

Installation is described in the README.txt file.

OVERVIEW

After installing the ScAnVP software, the ScAnVP menu can be invoked

by typing 'scanvp' in the MATLAB command window.

ScAnVP tasks are initiated by pressing buttons or selecting

options from the submenu controls on the main menu.

The default directory is the MATLAB Current Directory and can

be changed using the MATLAB desktop tool or ScAnVP menu item.

The main function of major menu buttons is briefly described below.

More detailed information can be obtained for each task by setting

the Help Mode switch ON on the main menu and clicking on the task.

This also works for some intermediate menus. In addition individual

'HELP' buttons are also available on task menus after initiation.

Tooltip strings are provided to clarify button functionality.

Images must be in ANALYZE format or Nifti format with some exceptions.

Conversion routines are provided to convert GE '.iee' format PET images

as well as GE MRI GENESIS format and Nifti images to ANALYZE format.

Unix ANALYZE image headers must be converted using the UNIX<=>DOS utility.

Functions are provided to convert raw count data to glucose rates,

perform regional graphic analysis or create voxel based Logan plots and

Patlak maps.

Display routines allow interactive visualization of orthogonal multiple

slices with different color schemes, interactive thresholding, voxel

value interogation, area and line value plotting and cluster removal.

Region of Interest (ROI) routines are provided to create a template of

rectangular, irregular or elliptical shaped regions on sequential

slices of a volume image. Standard or user-defined names can be assigned

to these regions and interactive options are provided for manipulation,

thresholding, statistical tabulation and mapping. The ROI template output

files can be input into group replay processing routines to create output

tables of statistical 2D and 3D ROI group value summaries in various formats.

Alternatively an AAL_MNI BRAIN ATLAS of predefined voxels is provided.

Information from these tables can also be input into ROI-based statistical

analysis routines. PCA covariance patterns derived by ROI-based SSM

principal component analysis can be visualized using the GIS ROI voxel

map routine. More typically, voxel based SSM analysis is used to study

covariance structure that is not limited by predefined regional boundaries.

Group subject files are initially transformed into a standard spatially

normalized space such as the MNI space defined in SPM normalization.

SSM voxel based results can be visualized using the statistical mapping

routine that also provides thresholding, masking and cluster removal.

SPM-like results of voxel-based SSM output is an additional option.

ScAnVP also provides various image utilities and processing routines

including functions for transparent fusion of PET with coregistered

MRI images, group plot/files of cubic and spherical region values, and the

ability to save and display image files in other standard formats.

Selected documentation can be found under Revisions/Notes.

MENU

A brief description of menu items is given here. Further details

can be found from individual menu help.

Menu options are divided into the following categories:

CONVERSION, FUNCTIONALIZATION, DISPLAY, REGIONS OF INTEREST, STATISTICS, IMAGE PROCESSING, UTILITIES, HELP & CONTROLS

CONVERSION:

GE(PET)->ANA

Converts GE PET, iee image files to ANALYZE format.

(128x128, 8bit, Radiological (RL) orientation, zero negatives).

GE(MRI)->ANA

Converts GE MRI, GENESIS image files to ANALYZE format.

(256x256, 8bit or 16bit Radiological (RL) orientation ).

UNIX<->DOS

Converts ANALYZE image headers from UNIX to DOS format

or vice-versa. If voxels have greater than 8bits, image

files are also converted. Multiple Volume images are

separated into single volume image files.

NIFTI->ANA

Converts spm5 'nifti' image headers to ANALYZE format.

An option is provided to flip the image along the X Axis.

This enables Radiologic Oriented images (RL) output by SPM

to be converted to Neurological Orientation (LR, Left is Left).

FUNCTIONALIZATION:

FUNCTIONALIZATION

FDG Glucose - Blood File Input

Converts raw count Analyze images to Cerebral Metabolic

Glucose rates (mg/100g/min).

FDOPA Uptake - Blood File Input

Converts raw count Dynamic Analyze Images to Uptake

units (micro liters/g/min).

GET ROI PARAMETERS - Roi File Input, Varous Tracers

Performs ROI based graphic analysis- Produces SOR, Ki

and DVR Plots and an output text file.

GET VOX PARAMETERS- Roi File Input, Various Tracers

Creates voxel based Logan plots and Patlak maps saved as

ANALYZE DVR and Ki parameter images.

DISPLAY:

IMAGE VIEW

Displays images in 3 orthogonal views and in sectional tile formats.

Image voxels can be bytes, integers, or real float or double

and may include negative values.

Images are displayed in Neurological orientation (Left axial

display corresponds to the anatomical left hemisphere) when the

saved orientation in the ANALYZE file saved on disk is correctly

specified by the user on the ScAnVP menu as either Neurological

(LR) or Radiological (RL) Input Orientation. Newer raw axial

images are usually saved in the radiological convention (RL):

Right display is left of subject and left is right, for example,

GE GENESIS-MRI and IEE-PET format output images. SPM usually

displays images in Neurological orientation, however, on disk

spm5 normalized images are saved in radiological orientation

while spm99 normalized images are saved in neurological orientation.

Coordinates:

Scanvp display routines such as 'IMAGE VIEW' calculate physical

coordinates based on the voxel coordinates of the origin

and the physical dimensions of each voxel. If the images

are normalized, the physical coordinates of each point

represent the distance in mm. from the normal origin.

Using the SPM normalization routine, physical coordinates

represent coordinates in a Talairach or MNI based normal space

assuming the images are displayed in Neurological orientation.

For Radiologic oriented images, the positive X Axis corresponds

to the negative Talairach axis. In ScAnVP, images specified as

RL are automatically flipped L<=>R for display only to enable

proper coordinate and anatomical labeling.

Plot:

A plot option creates a 3D plot of a rectangular area's intensities.

A 2D plot option plots a line segment profile.

Rotate:

The rotate view option allows sectional views of a plane rotating

around an axis parallel to the X,Y or Z axis to be displayed and

animated (movie).

Crop/Zoom:

Images can be cropped and zoomed in any orthogonal view and saved.

Clusters:

The Del Clusters options allows the user to remove 2D/3D isolated

clusters less than a given diameter from an image or composite

PET/MRI map.

Dynamic/Multiple Images

Sectional slices from multiple images or dynamic sets of the same

subject can be displayed in various formats.

TRANSPARENCY

Works similar to 'IMAGE VIEW' but displays superimposed PET/MRI views

transparently. The user can vary the degree of transparency.

UTILITIES

Various utilities can be invoked from the main menu or from

a control bar on top of image figures. They include operations for

smoothing, reading a value of a point on an image, text placement,

colormap selection and colorbar placement, colormap manipulation,

thresholding and normalization of multiple image figures to a common

Min/Max range, deletion and movement of component elements, line and

surface plot creation, saving MATLAB figures in various other

standard image formats and displaying different format images as well

as previously created movies in a MATLAB figure.

COLOR

Allows interactive selection of a color map for application

to a data figure. A separate 'col' directory is provided for

saving or retrieving user color maps. Screen background GUI.

RANGE

FULL: Interactive variation of the upper and lower thresholds

and center of the colormap of a data image.

SEMI: Separate manipulation of the upper and lower half of the

colormap

NORM: Normalization of all images in a figure to a common scale.

VALUE

Allows the user to select a point on a data image and display

its value and coordinates in voxel and physical units. It also

displays the default Neurological ATLAS label designation.

ScAnVP displays images in the Neurological orientation assuming

their input orientation is properly specified.

TEXT

Allows the user to place text at a point on a data image

MOVIE

Plays previously created movies of a sequence of MATLAB image

frames saved in a .mat file. The 'IMAGE VIEW' ROTATE option can

be used to create movies.

Marker

Places a fixed size (default 8mm) geometric marker at a

specified or pointed-to location on a displayed image.

REGIONS OF INTEREST:

ROI (Template)

This routine allows the user to interactively define

rectangular, rlliptical or irregular regions of interest

on sequential slices of a single subject image file.

The regions are named using standard or user names.

Statistical data can be tabulated and viewed and mapped.

Thresholding and editing options are provided. The data

is saved in files (Name.Plane.roi) that contain the

statistical and geometric information. These files can

be used as region definition templates for group processing.

GROUP-ROI (Template and ATLAS ROIs)

Various options are provided to replay .roi templates on group

member normalized image sets. Three dimensional thresholding

options and interactive editing is included. Output can be

saved in several different formats for individual or summary type

reports. In addition to text .roi templates the program can use

as input an image roi ATLAS where each brain area corresponds to

a coded voxel value listed in a corresponding label index file.

The default atlas file may be referred to by other routines.

This atlas was originally based on the aal_MNI_V4.img and can be

used on any images spatially normalized to MNI space 79x95x68 or x69.

MASK

Uses a reference image to allow the user to interactively create

a 0/1 8bit output ANALYZE format image mask across planes with

values of one for every voxel that is included within regions

and zero for voxels excluded.

VOI PLOT/SAVE

Gets values for specified points or average values within 3D

spherical or cubic volumes of interest for a set of image files

and plots or saves the values in a file.

STATISTICAL MAPS:

STATMAP

Utilizes Analyze format statistical output, Mask and Background files,

to create a masked, thresholded, statistics over background file

with exclusive and inclusive voxel selection and various orthogonal

viewing options similar to 'IMAGE VIEW' including 2D and 3D cluster removal.

GIS ROI MAP

Creates a statistical voxel map of a regional GIS PC vector

resulting from SSM analysis of ROI template group data. Options

are provided to manipulate the visual representation over an MRI

background image (PET/MRI). Output is saved in ANALYZE format.

GIS ATLAS MAP

This routine creates an ANALYZE image map of GIS ROI weight

output from SSM ATLAS GROUP ROI analysis. The voxel output image

can be input into the 'Statistical Map' routine to create

a PET/MRI image map.

SPM-like RESULTS

Provides an interactive gui for displaying SSM or other voxel

based statistics similar to that produced by SPM results.

GROUP MASKS

Group threshold masks (0|1) images can be created and applied in

various applications. A default gray matter mask (PETmask.img) along

with several other masks are provided under the scanvp\masks

directory. This default mask can be replaced by any user mask provided

that it has the same dimensions as images used in processing. Provided

are masks labeled with the characters 99 or 5 to indicate the version

of spm used to normalize the files that were used to generate the mask

and the intended version of normalized images for which it should be used.

In this version the default mask is symmetrical and compatible with spm5

(identical to the label '5' mask which has 69 planes).

RATIO MAPS

Creates an image from the corresponding values of two sets of images.

STATISTICS:

SPM

Invokes the SPM MATLAB package when available.

SSM   (See SSM HELP and DOCUMENTATION)

Includes several options for both VOXEL and ROI based SSM-PCA.

PCA

SSM Principal Componant Analysis (PCA) is used for the derivation

of group invariant subprofiles (GIS networks) and associated

subject scores, These network PC patterns may be significantly

related to disease specific metabolic or other physiological

processes. Subject scores for these patterns determined by the

algorithm can be used in differential diagnosis, patient monitoring

and drug performance evaluations.

SUBTRACTION-PCA

Principal Component Analysis of dual condition subtraction data.

For log transformed data (the default) this is equivalent to a ratio

operation.

O-PCA

Orthogonal PCA analysis (O-PCA) of data where the covariant

contribution of a particular GIS component is removed

to enable other components to be revealed.

TPR

Topographic Profile Rating (TPR) of new subjects for the

determination of disease specific GIS Scores that evaluate

disease severity on an individual, prospective basis.

Both highly automated and interactive procedures are available

for reference based scoring and reference recalibration.

Repeated measures for independent and same group repeated

trials are saved in text files and displayed as scatter plots.

UTILITIES:

DISPLAY SCORES

Displays bar and scatter graphs of subject scores for SSM GIS score files.

GENERATE MEAN

Creates Voxel Based Group Log Mean Image for SSM Statistical Analysis.

ROC CHARACTERISTICS

Displays the ROC characteristics of an SSM score file.

GROUP THRESHOLD MASK

Creates Voxel Based Data Threshold Mask from Group Data.

EXCLUSIVE MASK

Creates Voxel Based Exclusive Mask from Data or GIS File.

MASK GROUP DATA

Masks Data Images Using Pre-Defined Mask

LINEAR GIS ADDITION

Creates a Composite Voxel GIS Pattern From More Than One PC.

GIS CORRELATION

Calculates Pearson's Correlation of a Set of GIS or Other Image Vectors

IMAGE PROCESSING:

Options include SINGLE, MULTI, or PAIR-WISE operations.

SINGLE operations act on individual images within a selected group

separately. MULTI operations act on a group of images to produce

a single output image. PAIR-WISE operations act on pairs of images

from two corresponding sets.

Operations include SCALAR and IMAGE BASED arithmetic, image SCALING, CROPPING, GLOBAL or VECTOR NORMALIZATION and Z-SCORE conversion.

Some of the other available options are listed below.

APPEND

Concatenates ANALYZE images into one by appending component planes.

AVERAGE

Scales voxel values and averages component image planes.

ADD

Scales voxel values and adds component image planes.

SUBTRACT

Scales voxel values and subtracts from the first image. If negative

values occur, the resulting image is saved as same type if possible.

DIVIDE

Scales voxel values and divides into the corresponding voxels of the

first image. For zero voxels the result is set to zero.

EXTRACT SLICES

Extract a range of slices from an ANALYZE image to create a new file.

Combines two or more slices within a file into a sigle slice file.

CONVERT TYPE

Converts image TYPE. For example, a 16bit image to 8bit and vice-versa.

FLIP AXIS/VIEW

Flips X, Y or Z axis or converts from Axial to Sagittal or Coronal views.

INTERPOLATE

Change image dimensions using tri-linear or nearest neigbor interpolation

and save new file.

SMOOTH

Smooth image using user specified box convolution kernel.

MARKER

A square marker is placed at the same position in each plane.

NEGS.->ZERO

Sets all negative values to zero.

EXCLUDE VOX

Excludes voxels from a file that are common to a reference.

HEMI IMAGE

Display only Right or Left Side of Image.

MULT, THRESH, MASK, LOG, GLOBAL NORM...

MULTIPLY BY SCALAR

Multiply image values by floating point number.

THRESHOLD LOWER VALUES

Set values less than a lower threshold to zero.

EXCLUDE THRESHOLD RANGE

Set values within given range to zero.

MASK

Set values to 0 in image corresponding to 0 in 0|1 mask

GLOBAL NORMALIZATION

Values are masked and then thresholded by setting the lower

threshold to a fraction of the image maximum within the masked

values. The values are then divided by the remaining non-zero

voxel mean.

MASK CONVERSION WITH EXCLUDED RANGE

Set values within given range to zero and convert to 0|1 mask.

LOGARITHMIC CONVERSION

Convert image values > 0 to their natural logarithm.

All other values are set to 0.

HELP:

HELP ON/OFF

HELP ON/OFF is a button which operates like a switch on the ScAnVP menu.

When ON, help is displayed in the desktop HELP or MATLAB command window when a task is picked on the ScAnVP main menu. The help is the description of the MATLAB routine found at the start of each .m file. The information may include the time the original version of this routine was created.

When OFF ScAnVP tasks run in the normal mode.

In addition, many button options provide a 'TooltipString' to clarify

their use. A 'help' button is provided on many application menus as well.

This button is not a switch and is not intended for use while processing.

GENERAL

A general description of ScAnVP features and routines (this file).

REVISIONS

A file listing major revisions of each new version of ScAnVP.

NOTES

A file listing revisions for versions of ScAnVP prior to 5.0.

DEFAULT NAMES

A file listing default naming conventions for output files.

ANALYZE HDR

Lists header parameters of a user selected ANALYZE file in the

MATLAB command window.

FIX HDR

Displays header parameters of a user selected ANALYZE file and

allows modifications to be made and saved. New header files can

be created. Global variable defaults for ANALYZE Headers are

initialized in the vp_defaults.m file.

FIX TIMES

Alows the user to save time fields from a set of ANALYZE headers in a

file or to restore time data from a file.

FIX ORIGIN

Alows the user to correct the X Y Z voxel coordinates of the

origin of ANALYZE image headers.

ERRATA

Lists known problems with ScAnVP routines that need to be addressed

or that have been recently fixed.

CONTROLS:

RESTART

In case of an operational error, press RESTART. The RESTART button

will abort tasks, clear the screen and restart ScanVP.

Also used to set the file browser to the MATLAB current directory.

MESSAGE

Displays the last ScAnVP status message and last MATLAB error

when the user clicks on the latter field.

CURRENT DIRECTORY

Invokes the MATLAB file browser and allows the user to

change the current directory and view listed files.

The file selection browser defaults to the MATLAB current

directory on start up. It serves as a default output directory.

QUIT

Aborts tasks, clears the screen and exits ScAnVP

___________________________________________________________________________________

vp_general.m  ScAnVP 7.0w        Phoebe Spetsieris, Ph.D. Jan. 31, 2016

Phoebe G. Spetsieris, Ph.D.

Research Associate Investigator

Center for Neurosciences

Functional Brain Imaging Laboratory

The Feinstein Institute for Medical Research

Northwell Health

pspetsie@nshs.edu