Cropping two SPATIALLY normalized images that have different dimensions

Assume you have two images that are normalized, coregistered with the same voxel size but different dimensions. For example one may be padded outwardly with zero space. We assume that the origins of the images correspond to the same (0, 0, 0)mm physical coordinate of registered space (e.g. MNI space) but will have different voxel coordinates.

Large Image:  Dimensions DX2, DY2, DZ2, Origin: x2, y2, z2

Small image:  Dimensions DX1, DY1, DZ1, Origin: x1, y1, z1

To crop the large image to the space of the smaller image, we must specify the two diagonal points P1 and P2 defining the smaller volume within the space of the larger image volume.  Planar x,y,z coordinates are specified here as voxel coordinates within the native  disk volume. Disk file Left/Right anatomical orientation may not matter if the crop operation is symmetrical or the image is in Neurological (L/R) orientation.  Otherwise, for Radiologically oriented images, the user can re-evaluate the required xcoordinates as  (DX -x), where DX is the Axial Voxel Width of the image :

...……………………………………………………………....(DX2, DY2, DZ2)

…………………………………….……..P2(x’, y’, z’)

|                       Origin(x2, y2, z2)                |

P1(x, y, z)………………………………………………

(1, 1, 1)……………………………………….......................................

P1 (x, y, z), where x=x2-x1 +1, y=y2-y1+1, z=z2-z1+1  (We add 1 because voxel coordinates begin with 1)

P2 (x’, y’, z’), where x’=x+DX1 -1, y’=y+DY1-1, z’=x+DZ1-1

To use the “slice crop” routine in “Supl. Image Ops” in scanvp, Select the “Crop Slices” option and enter the x, y corrdinates of P1 and P2 followed by the two z coordinates which are the slice numbers z and z’.

EXAMPLE:  Crop Large image DIM(91 109 91), Origin( 46 64 37) to DIM(79 95 69), Origin(40 57 26)

Calculate:

x=46 –40+1= 7, y=64–57+1= 8, z=37-26+1= 12, x’= 7+79-1=85, y’=8+95-1=102, z’=12+69-1=80

In the “Crop Slices” routine enter the following numbers and then process:

P1( x, y):  7 8

P2(x’, y’): 85 102

Slices z, z’: 12 80

Reverse problem – Pad images

Calculate difference in dimensions and divide by two. Use Slice-Pad option to pad  twice, in the negative, followed by the positive axis directions;

Example:  To pad an 79 95 68 image with Origin (40, 57, 26) to a 91 109 91 image with Origin (46,64,37):

Calculate: dx=91-79=12,  dy=109-95=14, dz=91-68=23

Pad negatively: (-6, -7, -11), Pad positively: (6, 7, 12)

P. Spetsieris  	 1/30/2016