SELECTED HELP TOPICS

ScAnVP ver. 7.0   1/31/16

This document contains help notes on a selected set of topics regarding ScAnVP function and operation. The programs, have been developed and validated at the Functional Brain Imaging Center for Neurosciences of the Feinstein Institute for Medical Research in the North Shore - LIJ Health System headed by Dr. David Eidelberg. Detailed Help is available on the ScAnVP menu as outlined below. Only a few general areas are addressed here. A separate document written by Dr. Yilong Ma contains instructions on the application of multivariate SSM-PCA techniques in neuroimaging analysis. Additional references pertaining to theoretical concepts of SSM, applications and how-to details are included in the end of this file.

A. ScAnVP Installation steps

- Click on the compressed archive ScAnVP7.0.zip or if previously expanded, copy the scanvp\ directory to your selected location (e.g. at c:\scanvp).

- link only the subdirectory “vpm” directory path (e.g., c:\scanvp\vpm) to matlab with 'file', 'set path', 'Add'.

- At matlab prompt: type 'path' to confirm the setting.

B. Help Options

- Text and word format help is available under the scanvp\documentation directory.

- General information and help routines are available on the ScAnVP menu.

- For help on specific routines set the 'Help Mode' button ON and point to main menu or expanded menu tasks. This can also be performed at various interim interfaces within program execution.

- Run time help buttons are also provided.

- ‘Tooltip’ strings indicate the function of button options.

- See also references listed below and available on line

C. Data Types –

ScAnVP/SPM 	 MATLAB Type

TYPE 1:		bits/pix = 1;

TYPE 2:		bits/pix = 8;	uchar

TYPE 4:		bits/pix = 16;	int16

TYPE 8:		bits/pix = 32;	int32

TYPE 16:		bits/pix = 32;	float

TYPE 64:		bits/pix = 64;	double

D. Origin and XYZ Coordinate Display – Steriotactic Normalized Space

Scanvp display routines such as the imview routine calculate physical coordinates based on the voxel coordinates of the origin and the physical dimensions of each voxel. These values are read from the ANALYZE header and are stored in the MATLAB global variables ORIGIN and VOX. If the images are successfully normalized using spm99 or spm5 or spm8, the values are ORIGIN=[40 57 26] and VOX=[2 2 2]mm. For spm normalized images, the physical position of the origin should correspond to that defined by the normalized MNI space of the spm template which approximates the Talairach based template. When a properly normalized image is brought up by scanvp, the cross in the three orthogonal views should initially point to the Talairach or MNI space origin. Coordinates of other points are in mm from the origin and should correspond closely to Talairach values assuming the image is displayed in Neurological orientation (Left is anatomical Left and Right is anatomical Right) or (LR). If the images are in Radiological orientation (RL), the displayed X values correspond to –X in the Talairach coordinate system (anatomical left). For spm5 and spm8, output normalized images are unchanged from their original RL orientation. They are output in ‘nifti’ format which can be either .img, .hdr or .nii format. RL format images can be converted to Neurological LR orientation but this is not required for LR display in ScAnVP in the current version assuming RL input orientation is specified on the main menu. It is the left for the user to keep track of any changes in orientation.

You can use the transparency option to check overall correspondence with a Talairach/MNI normalized MRI image. Occasionally there may be a problem with spm normalization (registration) of some images if the original image space deviates too far from normal space. In that case you may have to define the approximate physical position of the origin using either spm or scanvp routines to be close to the Talairach origin before performing the normalization. The dimensions of spm99 normalized images are [79 95 68] and spm5, spm8 normalized images are [79 95 69]. These images are sometimes padded with zeros in the external space to allow for rotation in alignment. See document “Crop_Pad_Normal.doc” in scanvp\documentation on how to crop or pad various sized normalized images to conform to uniform dimensions.

E. Conversion of GE PET IEE or MRI images to ANALYZE format – Radiological  or Neurological format

Using the conversion routines, images are read as short values (16bit, 128x128x35 for PET) from the scanners data base and negative values are set to zero when they are initially stored as 8 bit or 16 bit ANALYZE images. Images are initially in the Radiological 'RL' orientation. Right is left and left is right but they are converted to 'LR' when they are normalized by spm99 assuming that the initial 'RL' orientation is set as a default parameter in spm99 or set by the user. Spm5 operates differently as noted below:

SPM99: For conversion from RL to LR in normalization the following line must be included in the spm99\spm_defaults.m file:

sptl_Ornt = [0 0 0  0 0 0 -1 1 1 0 0 0]; (Radiological Convention, Left is Right)

(If the files are initially in Neurological format (Left is Left) the -1 parameter should be +1.)

For spm5, there is a default flip value (defaults.analyze.flip) that has to be set to 0 in the spm5\spm_defaults.m file in order for images to be displayed in the Neurological convention in spm following normalization. A copy of this file is provided under scanvp\documentation. However their orientation is not changed when they are output in spm5. Therefore, ScAnVP in previous versions displayed spm5 normalized images in Radiological format unless they were ‘flipped’ LEFT  RIGHT on the X axis using the Convert Nifti to Analyze or the Flip X Axis routines. In the current version, ScAnVP keeps track of the input orientation specified on the main menu, and displays images in the Neurological orientation.

If in doubt, scanvp provides a 'marker' utility in which a bright marker is placed on the image that can be used to track changes in orientation during image processing operations for test purposes.

F. NIFTI to ANALYZE Read/Convert

Image files output from spm5 which are in 'nifti' format can be read and processed properly by most scanvp MATLAB routines or are flagged for conversion to ANALYZE using the provided option (convert NIF ANA). This routine automatically corrects for ORIGIN coordinates and voxel values that may be erroneously displayed in older routines. For most scanvp MATLAB routines (notably, Voxel SSM PCA), the conversion is performed transparently and is not necessary before processing except for .nii type images. The NIF ANA routine also provides an option for the user to flip the images along the X Axis for RL  LR conversion of Radiological to Neurological orientation or vice-versa. If images are both in UNIX and ‘nifti’ format, the NIF ANA scanvp routine will convert them to DOS ANAYZE. In cases where the image files display an incorrect origin in ScAnVP because of previous miss-handling, the ScAnVP FIX ORIGINS routine can be used to convert the ORIGIN to specified coordinates (e.g. 40 57 26 for spm Normalized files).

G. XYZ Coordinate Display

X, Y, Z coordinates represent mm distance from the origin (see also section on Origin and Coordinates). In ScAnVP, these values correspond to Talairach coordinates only if the file is normalized and displayed in the Neurological Orientation. If it is normalized in the Radiological orientation and so specified on the ScAnVP menu, it is flipped along the X axis for Neurological display, otherwise X corresponds to –X in the Talairach coordinate system for Radiologically oriented files. The brain regional names displayed by the point VALUE control refer to the saved Atlas reference image which is by default in the Neurological (LR) orientation and assumes that the display image is also LR oriented. See also the note above regarding orientation is spm.

H. Global Normalization (dividing by the subject mean thresholded value)

This is an optional image processing operation not required in SSM network analysis. If selected, each image’s voxel values are masked and then thresholded by setting the lower threshold to a fraction of the image maximum within the masked values. The values are then divided by the remaining non-zero voxel mean. Zero values are excluded in the calculation of the mean. The fraction can be 0 to include all positive values.

I. Automated SSM Network Reference File Naming

Automated SSM TPR options to generate prospective single-subject scores for new subjects following an initial SSM PCA reference group study, are enabled by using a standard directory structure and naming conventions for network reference files corresponding to output produced by the SSM PCA routine. For example, to allow automatic access using AUTO TPR to results of the SSM PCA analysis of study “Study” for the hypothetical condition “Cond”, the reference files would be placed in a directory named Cond under scanvp\networks. Files that must be included in the voxel network subdirectory for 'Cond' are:

EXAMPLE (Voxel Based Analysis)

Directory:	scanvp\networks\Cond\VOX\Study\

lnmean file:	Study_lnmean.img, Study_lnmean.hdr

GIS file:	Study_gis001.img, Study_gis001.hdr

Score file:	Study_VScores1.txt

Note, VOX can be named VOX5 or VOX99 to indicate spm5 or spm99 normalized files.

Files that must be included in the ROI network subdirectory for hypothetical condition 'Cond' are:

EXAMPLE (ROI Based Analysis)

Directory:	scanvp\networks\Cond\ROI\Study\

lnmean file:	Study_lnmean.txt

GIS file:	Study_gis2.txt.

Score file:	Study_RScores2.txt

The pre-assigned gis id (1 or 2 in the examples) can be edited by the user.

A summary report (Study_SUM(3D)_c.txt, instead of the 'lmean' file

can be provided but it can only generate Z scores in interactive mode.

J. SSM Masking and Threshold specification

For ScAnVP SSM PCA analysis, a mask (default) or threshold T must be specified. The program will consider T=.35 as the threshold default if it is not overridden. Images are assumed to be in normalized space. The common mask is applied to each image or the fractional threshold is applied to each image's maximum value to determine it’s lower threshold. For threshold selection, an overall mask image is created as a composite product of individual 0|1 maps of values above each threshold. If a pre-existing mask is specified, the threshold is set to 0 and the group output mask is the same as the pre-existing mask except that any voxels that are zero for any subject are set to zero. Subject derived masks are multiplied to create the final mask. Thus the output mask can differ from the input mask. T=0 is essentially the same as no mask assuming positive values and in this case a mask with all values set to 1 for non-zero voxels and 0 otherwise will be output. If you specify the SRP option for input, you can specify "No Mask" but the program will derive a mask from the SRP images themselves and combine it with any specified external map and then output the resulting mask (thresholding does not apply to SRP).

K. SSM TPR scoring

Single-subject SSM scores can be obtained prospectively for new subjects using TPR automated and interactive options. It is assumed that any new subjects that are tested for their expression of pre-derived GIS networks are scanned in a similar way and with the same radiotracer as the reference subjects whose data is accessed by the routines. The files automatically referenced by AUTO TPR should refer to previously derived GIS networks that have been validated at the home site. Automated output scores are offset in relation to the reference group mean scores. If you have any doubt about the reference group, you can perform an Automated or Interactive Recalibration procedure with a new reference group and regenerate reference files with the same GIS which can be saved in an assigned directory under scanvp\networks as described below. Alternatively, non-normalized direct scores that do not require a reference group mean file can be generated from a pre-derived GIS image using interactive options.

L. SSM Recalibration for use with TPR scoring

In view of the fact that there may be a shift in scores for an established GIS profile in a new setting or under a new scanner or scanning protocol, a recalibration procedure can be used to generate a new set of reference scores for a new group of subjects. The necessary recalibration steps are performed automatically using a provided automated option but in case of inadequate memory or other issues, the interactive procedure outlined below can be followed.

A) AUTOMATED RECALIBRATION – The user selects a new group of subjects and a previously defined reference GIS network for a particular study of a certain disease or condition. Validated networks may be listed under: scanvp\networks\COND\VOX\STUDY.

A new set of reference files including scores will be generated and saved in a new reference study directory.

B) INTERACTIVE RECALIBRATION –

1) Perform a 'mock' PCA on a new group of patients and controls using a pre-defined mask if preferred. Use the PCA generated output mask and 'srp' files for subsequent operations but ignore ‘gis’ output and score files.

Note:

A shift in AUTO-TPR scores can result from not using the same mask in the original PCA and in the recalibration. If we want to use a particular mask, we should input that mask instead of 'default' when doing the PCA. PCA will create an output mask along with the SRP files that is the actual one used in calculating the SRP. This mask may be different from the one originally input because of zero values in the input data. The PCA output mask should be used for any subsequent operations including the recalibration and AUTO-TPR reference scoring.

2) Select Interactive Recalibration on the SSM menu and input the srp files and mask created in step 1 and the pre-defined GIS.

3) Create a new scanvp\network\directory and save the pre-defined GIS file, output file _VScores1.txt of step 2, and _lnmean.img and mask output files of step 1.These reference files must be stored with a common study name (see above). Include a text note for subsequent users clarifying what type of images were processed.

4) Test that reference subjects produce the same score when AUTO-TPR is applied to the new reference set.

M. ROI

ROI Planar Templates: The ScAnVP ROI template routine interactively creates plane specific template text files (NamePlane.roi) containing statistical information including the geometric coordinates of the regions. The number of pixels within a ROI and the number of threshold pixels that are evaluated to calculate the mean value are given for each subject and plane in the .roi template files. These files can then be input in the Group-ROI routine that generates a variety of reports depending on the options you select. Regions are labeled using standard or user defined names.

Standard ROI Labels

2   Hemisphere		45  Posterior Temporal

5   Cerebellar Hemisphere	50  Anterior Cingulate

6   Vermis			51  Posteror Cingulate

10  Pons			54  Middle Frontal

15  Midbrain			55  Lateral Premotor

20  Basal Ganglia		56  Sensori-Motor

21  Caudate Nucleus		57  Superior Frontal

22  Putamen			60  Calcarine

25  Thalamus			61  Temporo-Occipital

29  Inferior Temporal		65  Cuneus

30  Hippocampus		69  Parieto-Occipital

35  Middle Temporal		70  Inferior Parietal

36  Superior Temporal	75  Paracentral

39  Orbito-Frontal		80  White Matter

40  Operculum			91-98  Other Stndrd. Roi

ROI ATLAS Image Templates: In a ROI BRAIN ATLAS image, regions are pre-defined on each plane using a fixed coded index value for each voxel within a given region type. The user defined names of the regions are listed numerically in a separate text file with the name AtlasNameLabs.txt that must accompany the atlas. Limit planes are user specified in the Group-ROI routine.

Group-ROI: This routine can be run on a single subject or a group of spatially normalized subject images (to assure a one-to-one correspondence in the coordinate space) to generate reports of regional statistical information based on user specified planar template reports or on a ROI whole brain Atlas image template file. Various options are provided including thresholding, interactive tweaking in subject space, and statistical report output format. The Group SUMMARY reports tabulate mean values for each region type for each subject or for the composite group. In addition, the SUBJECT (.sas) type option outputs subject reports summarizing the means and number of threshold pixels for each plane and each region separately. The COMPOSITE option for SUBJECT reports gives the mean values for each region type.

N: References

- J.R. Moeller, S.C. Strother, J.J. Sidtis and D.A. Rottenberg "Scaled Subprofile Model: A statistical approach to the analysis of functional patterns in positron emission tomographic data," J Cereb Blood Flow Metbol., 1987; vol.7, no.5, pp.649-658.

- G.E. Alexander and J.R. Moeller, “Application of the scaled subprofile model to functional imaging in neuropsychiatric disorders: a principal component approach to modeling brain function in disease,” Human Brain Mapping, vol. 2, pp. 1-16, 1994.

- D. Eidelberg, J.R. Moeller, V. Dhawan, P.  Spetsieris, S. Takikawa, T. Ishikawa, et al., “The metabolic topography of parkinsonism,” J. Cereb. Blood Flow Metabol., vol. 14, pp. 783-801, 1994.

- Ma Y, Tang C, Spetsieris P, Dhawan V and Eidelberg D. Abnormal Metabolic Network Activity in Parkinson's Disease: Test-Retest Reproducibility. Journal of Cerebral Blood Flow and Metabolism: 27: 597-605, 2007

- Eidelberg, D. Metabolic brain networks in neurodegenerative disorders: a functional imaging approach. Trends Neurosci 32 (10), 548-557 (2009).

- Spetsieris, P. G. & Eidelberg, D. Scaled subprofile modeling of resting state imaging data

in Parkinson's disease: methodological issues. Neuroimage 54 (4), 2899-2914 (2011).

- Peng S, Ma Y, Spetsieris PG, Mattis P, Feigin A, Dhawan V, Eidelberg D. Characterization of disease-related covariance topographies with SSMPCA toolbox:effects of spatial normalization and PET scanners. Hum Brain Mapp, 2013; May 14, doi: 10.1002/hbm.22295. www.ncbi.nlm.nih.gov/pubmed/23671030

- Spetsieris P, Ma Y, Peng S, Ji Hyun K, Dhawan V, Tang C and Eidelberg D. Identification of Disease-Related Spatial Covariance Patterns Using Neuroimaging Data. JoVE, 2013.

Phoebe G. Spetsieris, Ph.D.

Research Associate Investigator, Center for Neurosciences

Feinstein Institute, Northwell Health

Email: pspetsie@northwell.edu