Neurological vs. Radiological Orientation

ScAnVP ver. 7.0   1/30/16

ScAnVp operates on Analyze format images and most Nifti images in any orientation. Reference templates and spatially normalized images may exist on disk in either Neurological (LR) or Radiological (RL) orientation but newer files are usually Radiologically oriented. Thus spm99 normalized files are usually LR whereas spm5 files are RL. A ‘LR’ or ‘RL’ specification is sometimes designated on ScAnVP files to clarify orientation. However, it is up to the user to verify that the correct input orientation is designated on the ScAnVP menu before display and for some processing operations. The Right/Left designation can affect the results of operations on images that involve specific anatomical position, e.g. Hemi_image, Asymmetry Index, VOI  values, etc. but does not affect processing that does not depend on orientation such as Global normalization or tasks that only require consistent orientation of group file input such as SSM-PCA.

Display Orientation Conventions

The following image display conventions are used as defaults by ScAnVP routines in the current version:

The image display functions display Neurological orientation (LR, anatomical right is displayed right) for both Radiologically and Neurologically stored input files assuming that the user specifies the correct disk input file orientation using the switch provided on the main scanvp menu. Files are usually saved in Radiological (RL) orientation (right is anatomical left) on the disk but displayed in Neurologica  orientation by the ScAnVP software and thus are flipped internally for  display only with respect to the X axis. Older images (e.g. spm99 normalized) are saved in Neurological orientation and the input setting should be set to Neurological to avoid erroneous flipping. The initial default input file orientation is Radiological for the current version of scanvp to comply with MRI and newer PET scanners. However, the user can overide the specification and it is saved until changed. For files normalized to the Talairach or MNI atlas, X coordinates correspond to Talairach/MNI coordinates in the Neurological oriented display. If the input is set to Neurological, files will be displayed in their native disk orientation regardless of the display setting as in previous versions of ScAnVP. (In Radiological display, X=-X Talairach.) Any new images generated by scanvp that depend on orientation are output  on disk in the same orientation as that specified on the menu as the input orientation unless otherwise noted.

SEE ALSO FURTHER INFORMATION IN SELECTED TOPICS

Prepared by:

Phoebe G. Spetsieris, Ph.D.

Associate Investigator, Center for Neurosciences

Northwell Health

Email: pspetsie@northwell.edu